1:	Findings derived from Inspection Check List:

| 1108 | Are accidents incidents and near misses investigated systematically onboard? There was no evidence available to illustrate that the ship’s crew and officers conduct investigation into reported onboard accidents and near misses. |  |  | N |  |
| --- | --- | --- | --- | --- | --- |
| 1109 | Is there objective evidence of company follow up of accidents/incidents, near misses? In connection with the above finding there was no evidence of company follow up |  |  | N |  |
| 1118 | Is Risk Assessment undertaken onboard? Despite the company SMS having a clearly written Risk Assessment policy, there was no record available to evidence any Risk Assessment activity onboard |  |  | N |  |
| 1144 | Is the ship's ETB (Emergency Towing Booklet) available - SOLAS Chapter II-1/3-4 The Master was not aware of this recent requirement and immediately contacted the Company to ensure that this finding would be rectified as soon as possible |  |  | N |  |
| 1145 | If the vessel is likely to transit the area where the risk of attack by Somali based pirates is considered to be high, is a copy of the latest version of BMP onboard along with NP Q6099 "Anti-Piracy Planning Chart"?" The vessel has regularly transited the Gulf of Aden but there was no copy of the current version of BMP onboard. |  |  | N |  |
| 2103 | Are Master's Standing Orders available? Apparently due to an oversight, no Master’s Standing Orders had been posted. The Master rectified this situation before the end of this inspection |  |  | N | R |
| 2104 | Are the Master's Night Orders available and in regular use? The Master re-instated such a system before the completion of this inspection |  |  | N | R |
| 2105 | Are all above countersigned and understood by all bridge officers? All deck officers were asked to read and sign the Master’s/Night orders and upon inspection, all signatures were present by the end of this inspection |  |  | N | R |
| 2129 | Are navigation shapes available? The vessel does not have the correct shapes onboard to be displayed by a vessel ‘Not Under Command’ |  |  | N |  |
| 2139 | Are procedures to vacate anchorage due to impending bad weather available? The SMS does not provide any specific guidance to the Master/crew on the subject of the vacating of an anchorage when heavy weather is forecast/expected |  |  | N |  |
| 3202 | Are portable oxygen and gas analysers provided and regularly calibrated? The ship has 2 oxygen/gas meters for use for enclosed space entry purposes. One of these units was without a valid calibration certificate |  |  | N |  |
| 3203 | Is adequate lighting provided throughout the vessel? The paint store light was found with an expired bulb. This was corrected before the end of this inspection. |  |  | N | R |
| 4210 | Is the hospital and bathroom clean and tidy? The Hospital is currently being used as additional storage and is not easily accessible |  |  | N |  |
| 5101 | Is a Class Survey Status Report available; is the ship free from CoCs / significant recommendations? The most recent quarterly listing was not available onboard. It was not possible for the Master to cross check whether Class records corresponded with his own survey file. |  |  | N |  |
| 5323 | Standard Marine Communication Phrases Was not available onboard |  |  | N |  |
| 6134 | Are machinery guards in place in the workshop and PPE available? Safety guard on engine room workshop pedestal drill found damaged and partially missing |  |  | N |  |
| 8115 | Is effective hazard marking employed (use of highlighting paint/contrasting colours)? There was no evidence onboard of the use of hazard marking in areas where there were head-height or tripping hazards. |  |  | N |  |
| 9107 | Are vents and air pipes on deck in satisfactory condition? FPK Tank vent pipe found to be holed 3 inches above the deck |  |  | N |  |
| 9208 | Are manhole covers in satisfactory condition? 4p double bottom tank manhole flange found to have 3 studs missing |  |  | N |  |
| 10004 | Are water ingress alarms in satisfactory condition and functional? At inspection a fault was showing on the alarm display panel. Rectified by shore technician prior departure |  |  | N | R |
| 10039 | If any Nickel Ore cargoes from Indonesia or the Philippines have been carried since 31st January 2011, can the Master confirm that the requirements of Club Circular 5:428 were complied with? The vessel has carried 2 such cargoes within the specified time period, however; the Master was not familiar with and was therefore unable to demonstrate, that the requirements of the circular were complied with on either occasion. |  |  | N |  |

2:	Best Practices Identified:

2136 - The SMS provides for a detailed section on the Voyage Data Recorder including use and maintenance requirements. The vessel’s Emergency Guidance Manual also includes this equipment in various casualty sections and acts as a reminder to the Master post-incident, to retain the recorded data.

3:	Other findings and comments made by the inspector to assist the member in Loss Prevention

None

| INSPECTOR |  |
| --- | --- |
| COMPANY |  |
| Date |  |